COB1144	INFLUENCE OF STEADY VERTICAL WIND PROFILES ON WIND TURBINES CYCLIC LOADING AND STABILITY

Janardan Rohatgi, Issac Barbosa Viera Filho

Departamento de Engenharia Mecânica, Centro de Tecnolgia e Geociências, Universidade Federal de Pernambuco, CEP 50.741-000 Recife, Brasil - E-Mail: janardan@npd.ufpe.br

This paper analyses the influence of steady wind shear (vertical wind profile) on the wind turbines structural stability. The wind speed, in general, increases with altitude (up to about 500 to 1500 m) above the earth’s surface. However, for wind turbines, the altitude is limited to about 120 m at the most because beyond this level the cost of tower becomes prohibitive. It should be emphasized that we are talking about the wind turbine’s rotor diameter of the order of 15 m or more. Hence, the wind speed at the extreme points of the rotor disc, considering perpendicular to the upwind would certainly be different magnitude, results into wind shear. The latter would cause cyclic load 1/revolution. This paper describes very briefly the equations of blade motion that determine the dynamic stability. This equation then is used to estimate the load due to wind shear. We have assumed a linear shear profile. The effect of it is estimated to calculate the bending moments. A step by step procedure is shown so as to have a real feel of the complexity involved in it.

Keywords: Wind Energy, Wind Turbines, Dynamic Stability, Wind Shear, Cyclic loading.