COB1042	ESTABILIDADE TÉRMICA DAS LIGAS AMORFAS Fe(85-X)B15GeX PROCESSADAS POR “MELT-SPINNING” / THERMAL STABILITY OF THE AMORPHOUS Fe(85-X)B15GeX  ALLOYS PRODUCED BY “MELT-SPINNING”

Carlos kleber N. Oliveira1 & Severino Jackson Guedes de Lima2

1Departamento de Engenharia de Produção, CCT, URCA,  Rua Cel. Antônio Luiz-1161,Pimenta, fax:(088) 521 0049,

63.100-000 - Crato-CE, Brasil.

2Departamento de Tecnologia Mecânica, CT, UFPB, E-mail: jackson @ vm.npd.ufpb.br , fone/fax: (083) 216 7076

58059-900 - João Pessoa-PB, Brasil.

Amorphous alloys Fe(85-X)B15GeX (x = 0-10) in the shape of ribbons were obtained by using the process of ultra-rapid “melt-spinning” solidification. The ideal processing parameters were determined and the alloys were produced in a compositions range, according to a theoretical diagram of amorphization compositions. The amorphous state was verified by X-ray difraction, Mössbauer spectroscopy and ductility tests. Differential scanning calorimetry (DSC) was used to determine the thermal stability of the alloys and observe the evolution of the crystallization peaks. Continuous amorphous ribbons were obtained reaching, in some cases, as much as 20m of length, presenting dimensional homogeneity and good superficial finish. The DSC thermograms showed the crystallization peaks which are characteristic of amorphous alloys, whose thermal stability rises with the Ge content.

Keywords: Amorphous alloys, “Melt-spinning”, Magnetic anisotropy, Amorphization and Crystallization.

Ligas amorfas, “Melt-Spinning”, Anisotropia magnética, Amorfisação e Cristalização.