COB780	ESTUDO DO ESCOAMENTO LAMINAR EM SISTEMAS AERO-SOLARES / LAMINAR FLOW ANALYSIS OF SOLAR-AIR SYSTEMS

Marco A. dos S. Bernardes, Márcio Fonte-Boa Cortez, Ramón M. Valle.

Departamento de Engenharia Mecânica, Universidade Federal de Minas Gerais

Av. Antônio Carlos, 6627, CEP: 31270-901, Belo Horizonte-MG, BRASIL - E-mail: marco@oraculo.lcc.ufmg.br

Solar Chimneys are devices used to convert solar energy into thermal and electrical energy. These devices are built with a transparent radial canopy covering the ground and a chimney positioned at its center. In the present work, the thermal hydrodynamic behavior of air in free convection in a Solar Chimney was theoretically analyzed to verify the geometrical and operational effects. The mathematical model formulation was performed considering laminar flow and prescribed boundary conditions. The visualization of the flow and the temperature profiles were allowed by the use of Finite Volume with Generalized Coordinates. Geometrical variations were introduced in a basic configuration to verify the system performance under variations of geometrical parameters. Three geometrical parameters were evaluated throughout simulation: the canopy/ground distance, the canopy/chimney connection shape and the chimney shape and cross section. The results indicated that the best performance was achieved with curved canopy/chimney connection shape and chimney conic shaped.

Keywords: Chaminé Solar, Volumes Finitos em Coordenadas Generalizadas, Convecção Natural, Energia Solar, Fontes Alternativas de Energia / Solar Chimney, Finite Volume Method with Generalized Coordinates, Free Convection, Solar Energy, Renewable Energy Sources.