COB1117	TRANSFERÊNCIA SIMULTÂNEA DE CALOR E MASSA NA FORMAÇÃO E ASCENSÃO DE BOLHAS SUPERAQUECIDAS / SIMULTANEOUS HEAT AND MASS TRANSFER DURING THE FORMATION AND ASCENSION OF SUPERHEATED BUBBLES

Flavio Barboza Campos e Paulo Laranjeira da Cunha Lage

Programa de Engenharia Química, COPPE - UFRJ - Ilha do Fundão,

CEP 21.945-970, C.P. 68502, Ilha do Fundão, Brasil - e-mail: paulo@peq.coppe.ufrj.br

A thermofluid dynamic model was developed to study the heat and mass transfer during the stages of formation and ascension of superheated bubbles in a liquid column. The gas phase physical properties and the bubble radius vary along the process, which makes this a free boundary problem. The model consists of the total mass, energy and chemical species conservation equations applied to a spherical bubble with radial symmetry, with gas injection at the bubble center during bubble formation, whose end is predicted using the final formation volume given by the classical Davidson and Schüler correlation. The internal radial convective flux and the interdiffusion term in the energy conservation equation were considered in the analysis. The model is solved numerically by the method of lines using finite volume discretization. The results obtained are analyzed and compared to a previously developed model restricted to the bubble ascension stage.

Keywords: Contorno livre, bolha superaquecida, evaporação, contato direto / Free boundary, superheated bubbles, evaporation, direct contact.