COB1132	CONJUGATE HEAT TRANSFER AND FLOW LOSSES IN COMPACT HEAT EXCHANGER WITH SMOOTH FINS AND TWO ROWS OF TUBES

Rogério Rodrigues Jr. & Jurandir Itizo Yanagihara

Department of Mechanical Engineering, Polytechnic School of University of São Paulo

Av. Prof. Mello Moraes, 2231 - São Paulo - CEP 05508 - 900  - E-mail: jiyanagi@usp.br

The present work refers to a numerical analysis of the local and global heat transfer and fluid flow characteristics of a fin-tube heat exchanger with smooth fins. These investigations aim to provide basic information for the development of heat transfer enhancement techniques applied to evaporator/condenser of domestic air-conditioning systems. Momentum and energy conservation equations were solved by the finite volume method with a boundary fitted coordinate system. The computational model consists of a compact heat exchanger with two rows of round tubes in a staggered arrangement using smooth fins. The numerical simulations were carried out for a steady-state thermally and hydrodynamically developing laminar flow. The influence of channel parameters such as transverse and longitudinal tube spacing on the heat transfer    and flow losses were analyzed. The results allow to identify situations in which the fin effectiveness decreases, providing useful information for the application of heat transfer enhancement techniques to this kind of geometry.

Keywords: compact heat exchangers; fin-tube channels; smooth fins; conjugate heat transfer; finite volume method