COB528	AN INVESTIGATION ON THE HEAT TRANSFER IN AN INDUSTRIAL FLUIDIZED BED HEAT EXCHANGER

Paulo Roberto Tardin Jr. and Leonardo Goldstein Jr.

Department of Thermal and Fluids Engineering, College of Mechanical Engineering - UNICAMP

Zip Code  13083-970 Campinas, Brazil - E-mail: tardin@fem.unicamp.br

Luis Carlos Casavechia

Superintendência da Industrialização do Xisto - SIX,  PETROBRAS S/A, Zip Code 83900-000 São Mateus do Sul, Brazil

An industrial size fluidized bed heat exchanger was tested and the results were applied to check the available correlations for the bed-to-tube heat transfer coefficient. The heat exchanger was designed to recover the heat content of the ashes produced in a 1 MW circulating fluidized bed boiler pilot plant burning oil shale, with an horizontal tube immersed in the bed. The solids inlet temperature was about 375 oC and the superficial velocity 0.10 m/s. About 75% of the total heat transfer rate from the ash was transferred to the water and the balance to the air stream. The Molerus correlation (1995) has shown the best agreement with the data, with less than 20% deviation.

Keywords: Heat exchanger, Fluidized bed, Heat recovery, Bed-to-tube heat transfer coefficient.