COB232	ANÁLISE E SIMULAÇÃO DE UMA TURBINA REGENERATIVA TIPO TOROIDAL / ANALYSIS AND SIMULATION OF A REGENERATIVE TOROIDAL  TURBINE

José Edmar Rios(*), Sebastião Varella(**) & Nelson Manzanares Filho(**)

(*)Departamento de Física Aplicada -  FUNREI - Praça Frei Orlando, 170,

CEP 36300-000 São João del-Rei MG, Brasil - E-mail: dfiap@csa.funrei.br.

(**)Departamento de Engenharia Mecânica - EFEI - Av. B.P.S., 1303

CEP 37500-000 Itajubá MG, Brasil - E-mail: nelson@iem.efei.rmg.br

An analysis and a simulation of a regenerative toroidal turbomachine are made in order to evaluate its potentiality as hydraulic turbine. The working principle of this kind of turbomachine is presented and discussed. Formulae developed on the basis of the circulatory theory are presented allowing the calculation of the circulatory velocity and the global pressure coefficient as a function of the main geometric parameters. Based on these formulae, simulation results were obtained for the  variation of the flow rate vs.  rotating speed with a fixed specific energy and taking  the diameter aspect ratio as a parameter. The operating characteristic is similar to that of a Francis turbine of high specific speed. In order to check the simulation model, some experimental results were obtained by means of rig tests performed on an hydraulic regenerative turbine. One verifies that the simulation model is capable of qualitatively reproducing the flow rate vs. rotating speed curve. One concludes in favor of the applicability of the regenerative turbomachine as  hydraulic turbine and suggests that new researches have to be made for further evaluations and improvements.

Keywords: Hydraulic Turbine; Regenerative; Toroidal. / Turbina Hidráulica; Regenerativa: Toroidal.