COB1404	ANÁLISE DO ESCOAMENTO IDEAL EM GRADES REGENERATIVAS TOROIDAIS COM EFEITOS NÃO-INERCIAIS / ANALYSIS OF THE IDEAL FLOW IN REGENERATIVE TOROIDAL CASCADES INCLUDING NON-INERTIAL EFFECTS

Ramiro G. Ramirez Camacho, Nelson Manzanares Filho & Sebastião Varella

Departamento de Engenharia Mecânica - EFEI - Av. B.P.S., 1303, CEP 37500-000 Itajubá MG, Brasil - E-mail: nelson@iem.efei.rmg.br

An ideal rotational flow model for regenerative toroidal cascades with thin blades including non-inertial effects is presented. A conformal mapping is applied in order to transform the toroidal cascade surface in a plane cascade. Vortex distributions are employed to simulate the blade perturbation in the transformed plane. The vortex intensity is calculated by numerically solving an integral equation representative of the relative flow tangency on the blades. The resultant blade circulation is related to the absolute vorticity on the physical toroidal surface. This vorticity creates the circunferencial pressure gradient of a regenerative turbomachine and this fact motivates the present research. Illustrative results for the vorticity and the slip factor are presented as a function of the main geometric parameters of the toroidal cascade.

Keywords: regenerative cascades; toroidal surfaces; non-inertial effects; vorticity. / grades regenerativas; superfícies toroidais; efeitos não-inerciais; vorticidade.