COB1425	ANÁLISE DO MÉTODO DE ORDENADAS DISCRETAS: DISCRETIZAÇÃO ANGULAR / ANALYSIS OF THE DISCRETE ORDINATE METHOD: ANGULAR DISCRETIZATION

Luís Mauro Moura, Dominique Baillis & Jean-François Sacadura

Institut des Sciences Appliquées de Lyon - CETHIL/TIM, ESA CNRS 5008 - 20, Av. Albert Einstein bât. 404 - 69621 Villeurbanne Cedex - France  E-mail: luis@ibjfs.insa-lyon.fr - luis@lmpt.ufsc.br

This work presents a numerical analysis of different sets of quadratures involved in a discrete ordinate method used to solve the Equation of Radiative Transfer (ETR). Two different quadratures are derived from the classical Gauss and Radau ones, with the imposed condition of satisfying key half-moments of the radiative intensity. This set of quadratures are obtained through a simple translation in the Gauss or Radau full-range scheme [-1,1] to the half-range [0,1]. These quadratures are compared to cases previously analyzed by Fiveland (1985, 1987) for a slab of isotropic conservative scattering or absorbing-emitting gray medium. The results obtained with the two new set of quadratures, for the net radiative flux and the incident energy to different range of optical thicknesses and the S2, S4, S6, S8, S10, S12 ordinate sets, show a good accuracy.

Keywords: radiação térmica, ordenadas discretas, meio semitransparente, quadratura

radiative heat transfer, discrete ordinate method, semitransparent materials, quadrature