COB1427	Prediction of directional spectral emittance of an absorbing and scattering ceramic material at high temperature

Rogério Lopes; Luís Mauro Moura; Agnés Delmas & Jean-François Sacadura

Centre de Thermique de Lyon - CETHIL/TIM - 20, Avenue Albert Einstein (bât.404)

69621 Villeurbanne Cedex - France - E-mail : rogerio@ibjfs.insa-lyon.fr

Directional spectral emittance of an absorbing and scattering isotherm ceramic material (Al2O3) is predicted using a radiative model based on the discrete ordinates method associated to the control volume technique to solve the radiative transfer equation. The absorption (Qa) and scattering (Qs) efficiencies for incident radiation upon spherical particles of aluminum oxide are calculated from the Mie theory for a temperature range from 1200°C to 2020°C, the particle radius lying from 10µm to 50µm, and a range of wavelengths from 2µm to 6µm. These calculations are based on literature values of the complex index of refraction (ñ = n - ik) at high temperature for the monocrystral of sapphire (Al2O3). Independent scattering assumption is used. The limit of independent scattering theory is verified for a particle volume fraction ranging from 0.3 to 0.7. The spectral normal emittance of Al2O3 at high temperature is calculated increasing the thickness to identify the 'infinite thickness'. Calculations are performed and analyzed for different particle radius and medium temperatures.

Keywords: Emittance, semitransparent media, ceramic material, high temperature, discrete ordinates method.