COB883	MODELO DE ESTIMATIVA DAS IRRADIÂNCIAS GLOBAL E DIFUSA EM COBERTURA DE POLIETILENO / MODEL TO ESTIMATE GLOBAL AND DIFFUSE RADIATION IN POLYETHYLENE COVERAGE

Valéria de Almeida Frisina & João Francisco Escobedo

Departamento de Ciências Ambientais/FCA/UNESP/Botucatu/SP, CEP 18603-970; CP 237; TEL (014) 821 3883; FAX (014) 821 3438

This paper describes the results obtained from correlations between global and diffuse radiation inside polyethylene coverage. During 40 days, global (G) and diffuse (D) radiation were measuread outside (ex) and inside (in) polyethylene greenhouse (100 mm), for four specter conditions: open sky, partially cloudy, cloudy and the sum of the three previous conditions. The global radiation was measured with pyranometers and the diffuse radiation, with pyranometer below shadow rings. The data aquisition system consisted of a CAMPBELL 21X datalogger and a 486 DX2 microcomputer, programed to operate in 1 Hz frequence and store average of 5 minutes. The following results were obtained: Gin = 4.13 + 0.79 Gex; Din = 45.53 + 0.71 Dex. The best correlations found to the global radiation (Gin x Gex) were, in order: cloudy sky days (R2 = 98.53%), open sky (R2 = 96.45%) and partially cloudy sky (R2 = 94.07%). The best correlations to the diffuse radiation (Din x Dex) were: cloudy sky (R2 = 96.52%), open sky (R2 = 78.93%) and partially cloudy sky (R2 = 71.78%).

Keywords: global and diffuse radiation; polyethylene greenhouse / radiações global e difusa, estufa de polietileno