# COB1289	ANÁLISE TEÓRICA DA CONVECÇÃO NATURAL LAMINAR NUMA CAVIDADE POROSA CONSIDERANDO O EFEITO DA POROSIDADE VARIÁVEL / THEORICAL ANALYSIS OF LAMINAR NATURAL CONVECTION INSIDE A RECTANGULAR CAVITY POROUS CONDIDERING THE EFFECT OF VARIABLE POROSITY

Jesus Marlinaldo Medeiros+*, Francisco Marcondes++ , Carlos Antônio C. Santos* & José Maurício Gurgel*

(+)Escola Técnica Federal de Sergipe/UNED - Aracaju - SE 49056-260

(++)Laboratório Computacional em Térmica e Fluidos - LCTF-DEM-CCT-UFPB - marconde@lctf.ufpb.br

(*)Laboratório de Energia Solar - LES-CPGEM-DTM-UFPB - cabral@les.ufpb.br - gurgel@les.ufpb.br

58059-900 João Pessoa - PB - Brasil

A numerical study is performed on steady two-dimensional natural convection inside a rectangular porous enclosure with uniform internal heat generation.  The boundary conditions were: Two isothermal walls at same temperatures, two horizontal adiabatic walls. The effects of non-uniform porosity are taken into consideration in the momentum equation. The governing equations in terms of the primitive variables are solved by the finite volume method. The coupling velocity - pressure was treated by PRIME algorithm employing staggered grids. The results are presented in terms of the isotherms, stream functions and Nusselt number with and without uniform porosity.

Keywords: Natural Convection - Porous Medium - Non-Darcian Model - Variable Porosity - Convecção Natural – Meio Poroso – Modelo não Darciano – porosidade variável