COB1228	ESTUDO DE TÉCNICA PARA SOLUÇÃO DE PROBLEMA INVERSO DE CONDUÇÃO DE CALOR BIDIMENSIONAL /STUDING OF TWO-DIMENSIONAL IHCP TECHNIQUES

# Frederico Romagnoli Silveira Lima & Gilmar Guimarães

Departamento de Engenharia Mecânica, Universidade Federal de Uberlândia, Bloco 1M, Campus Santa Mônica CEP 38-400 089 Uberlândia M. G. - E-mail: gguima@ufu.br;romagnol@ufu.br.

This work treats techniques for solving inverse heat conduction problems (IHCP) related to tools used in machining. The surface heat flux can be estimated from experimental and calculated temperature data. An evaluation of boundary conditions of metal cutting indicates the complexity of the thermal problem and a 3-D analysis is necessary. However, in a first study, the use of a simplified hypothesis enables two-dimensional (2D) analysis initially. The heat source due the machining process was simulated numerically to permit a great flexibility in the analysis of the technique used. The results of the 2D analysis are presented in several graphics. In this case, the linear imposed heat transfer history varies with time and position and the heat appeared to be lost laterally. The evaluation of results indicates the cases that the methods can be applied successfully.

Keywords: Inverse problems – parameter estimation – heat conduction – two-dimensional – cutting.

Problemas inversos – estimação de parâmetros – condução de calor – bidimensional – usinagem.