COB727	SOBRE A ESCOLHA DE SISTEMA DE CONTROLE DO VETOR EMPUXO DE MOTORES OGUETE A PROPELENTE LÍQUIDO / ON THE CHOOSE OF THE THRUST VECTOR CONTROL SYSTEM OF LIQUID PROPELLANT ROCKET ENGINES

Alexandr Alexandrovich Kozlov, Toshiaki Yoshino, Adalberto Pacífico Comiran, Mário Niwa

Divisão de Propulsão -- CTA-IAE

CEP 12228-904 São José dos campos, Brasil

This paper describes an analysis on the suitability of different types of thrust vector control (TVC) system to be applied to liquid rocket engines of open cycle scheme. It provides description of different types of existing Russian engines and the TVC methods that is utilized in these systems. In particular, detailed analysis is conducted on the propulsion system using liquid oxigen/kerosene propellants on three different open cycle configurations, aiming the second stage of VLS-2 launcher. These configurations are: a) one turbopump feeding four fixed combustion chambers, with the attitude control forces provided by four vernier nozzles expelling gases from the turbine; b) one turbopump feeding four fixed combustion chambers, with the control forces provided by four gimballing combustion chambers feeded by independent turbopump; and c) four autonomous gimballing engines, each one having its own turbopump and combustion chamber.

Keywords: propulsão líquida, motor foguete, controle de atitude, controle do vetor empuxo, veículo lançador

liquid propulsion, rocket engine, attitude control, thrust vector control, launch vehicle