# COB1094	Extraction of 3-D pore network and Prediction of Permeability for reservoir rocks

Liang Zhirong, Paulo Cesar Philippi, Celso Peres Fernandes & Fábio Santana Magnani

Laboratório de Meios Porosos e Propriedades Termofísicas de Materiais, Departamento de Engenharia Mecânica, Universidade Federal de Santa Catarina, 88040 - 900    Florianópolis,    SC, Brasil - E-mail: liang@lmpt.ufsc.br

The main purpose of the present work is to determine the 3-D pore structure network by using image analysis techniques and predict the permeability of the porous rock. The skeleton of a 3-D pore structure provides a way of visualizing the graph of the pore network. It is extracted using a thinning algorithm, which preserves connectivity, i.e., the network and original pore structure have the same topology. It gives both visual and quantitative information about the connectivity of the pore space, the coordination number for every node and local hydraulic radius. Once the network of pore structure is obtained, the macroscopic transport properties of the rock such as permeability can be calculated.    In this paper, the reconstructed porous media for Berea sandstone is used to extract 3-D pore network. The predicted permeability for 500mD Berea sandstone rock is in good agreement with the experimental value and empirical correlations.

Keywords: Porous media; skeleton; 3-D pore structure; permeability