# COB1093	RECONSTRUCTION OF 3-D PORE STRUCTURE BY the TRUNCATED GAUSSIAN METHOD USING Fourier transform

Liang Zhirong, Celso Peres Fernandes, Fábio Santana Magnani & Paulo Cesar Philippi

Laboratório de Meios Porosos e Propriedades Termofísicas de Materiais, Departamento de Engenharia Mecânica, Universidade Federal de Santa Catarina, 88040 - 900    Florianópolis,    SC, Brasil - E-mail: liang@lmpt.ufsc.br

The purpose of reconstructed porous medium is to simulate invasion processes of fluids and determine important petrophysical and reservoir engineering properties such as permeability, capillary pressure and relative permeability curves. In this paper, we improve the truncated Gaussian method for reconstruction of porous media using Fourier transform. The method is based on the idea that an arbitrary complex pore structure can be described by the values of a phase function at each point in the porous media. If the pore structure is statistically homogeneous, it can be described by the porosity and correlation function, which are measured from 2-D binarized image of thin section of the sample. If correlation function of Gauss field is positive-definite, corresponding Gauss field can be found. Since Fourier transform of correlation function is power spectrum of the field and the phase angle does not affect the correlation structures, then the Gauss field can be generated by using Fourier transform. An algorithm, which is a nice split-radix, n-dimensional and fast-Fourier transform is used. At last, the 3-D porous media are generated using the truncated Gaussian method. The result for Fontainebleau sandstone shows that the porosity and correlation functions reproduce well.

Keywords: Porous media; reconstruction; 3-D pore structure; image analysis; Fourier transform