COB886	MASS FLOW IN A TWO-PHASE FLOW WITH BOILING THERMAL SIPHON LOOP / VAZÃO MÁSSICA EM ESCOAMENTO BIFÁSICO COM EBULIÇÃO EM UM CIRCUITO DE TERMOSSIFÃO

Maria Eugênia Vieira, Hugo Leonardo de Brito Buarque & Públio Otávio Oliveira Duarte

Laboratório de Energia Solar, Centro de Tecnologia, Bloco 710, Campus do Pici - UFC

CEP  60455-760 - Fortaleza, Brazil  - E-mails: eugenia@les.ufc.br / hugo@les.ufc.br

This work presents a model to determine the total mass flow rate in a two phase flow with boiling closed circuit. This system is used to collect solar energy by the formation of vapor bubbles in a solar collector and to release this energy in condensation by heating a second fluid. The model assumes that the total frictional drop equals the pressure gain due to the difference in gravity in the single phase and the two phase vertical parts of the loop. Using the definition of the void fraction, a second expression could be written and the two equations were solved simultaneously for the void fraction and the total mass rate. The results indicated that the two phase flow was bubbly, what was expected considering the relatively low intensity of the solar flux and the experimental values for the temperature difference between the heated wall and the fluid saturation temperature.

Keywords: Thermal siphon, two-phase flow, mass rate, void fraction / Termossifão, escoamento bifásico, fração de void