COB1445	TURBULENCE AND PHASE DISTRIBUTION FOR SOLID/LIQUID TWO-PHASE FLOW WITH NEUTRAL BUOYANT PARTICLES

A. Assad1 and R.T Lahey, Jr2

1- Purdue University, School of Mechanical Engineering, West Lafayette, IN 47907-1077. E-mail amir@ecn.purdue.edu

2- Rensselaer Polytechnic Institute, Department of Energy and Environmental Engineering, Troy, New York 12180

Detailed local phase distribution measurements were made for solid/liquid two-phase flows in a horizontal pipe using a state-of-the-art laser Doppler anemometer (LDA) system. The disperse phase consisted of 2 mm neutral buoyant spherical particles. Since these particles had the same density as water particle/liquid slip was eliminated, thus simulating microgravity conditions. The conduit was a 30.6 mm diameter pipe, which was made of a special optically clear material (Fluorinated Ethylene Propylene) with the same index of refraction as water. A set of velocity profiles, turbulence structure and volume fraction distributions were measured for each phase. These experimental results were found to agree quite well with a multidimensional two-fluid model.

Keywords: Two-Phase Flow, Solid-Liquid Flow, Laser Doppler and Turbulence.