cob726	application of a sailing yacht velocity prediction program to A monotype and an oceanic sailboat

Mardel Bongiovanni de Conti

Rodrigo César Franceschini de Oliveira

Departamento de Engenharia Naval e Oceânica - EPUSP

Av. Prof. Mello Moraes, 2231 - São Paulo (SP) - CEP 05508-900 - E-mail: mbdconti@usp.br

This paper analyses the steady-state performance of an oceanic yacht cruiser and of a monotype sailboat. The first one is here considered with three sails, namely the main, the jib and the spinnaker, and the last one just with a main sail. True wind incidence angle is varied from 0 to 180 degrees. Three wind velocity amplitudes are considered: 9, 15 and 20 knots. The boat velocity and the leeway and heel angles are determined by solution of three non-linear equations. Two of these equations express magnitude equality and phase opposition between aerodynamic and hydrodynamic forces, and the third expresses equilibrium of heel moments. Suggestions for the first guess of the boat velocity, leeway and heel are made, in order to initialize the solution of the non-linear system. Some discussion about the shadow zone, the heel angles on close-hauled, the spinnaker rising effect and the true wind magnitude effect is presented.

Keywords: Naval Architecture, Yacht Design, Sail Propulsion, Sail Boat Performance