{\rtf1\ansi\ansicpg1252\uc1 \deff0\deflang1033\deflangfe1033{\fonttbl{\f0\froman\fcharset0\fprq2{\*\panose 02020603050405020304}Times New Roman;}{\f2\fmodern\fcharset0\fprq1{\*\panose 02070309020205020404}Courier New;}}{\colortbl;\red0\green0\blue0;\red0\green0\blue255;\red0\green255\blue255;\red0\green255\blue0;\red255\green0\blue255;\red255\green0\blue0;\red255\green255\blue0;\red255\green255\blue255;\red0\green0\blue128;\red0\green128\blue128;\red0\green128\blue0;\red128\green0\blue128;\red128\green0\blue0;\red128\green128\blue0;\red128\green128\blue128;\red192\green192\blue192;}{\stylesheet{\nowidctlpar\widctlpar\adjustright \fs20\cgrid \snext0 Normal;}{\*\cs10 \additive Default Paragraph Font;}{\s15\nowidctlpar\widctlpar\adjustright \f2\fs20\cgrid \sbasedon0 \snext15 Plain Text;}}{\info{\title TRANSONIC TURBULENT FLOW CALCULATIONS OVER THE VLS }{\author Ncleo de Apoio Computacional}{\creatim\yr1997\mo10\dy16\hr14\min22}{\revtim\yr1997\mo10\dy16\hr14\min22}{\version2}{\edmins0}{\nofpages3}{\nofwords471}{\nofchars2686}{\*\company UNESP/FEIS/DEM}{\nofcharsws3298}{\vern71}}\margl1319\margr1319 \widowctrl\ftnbj\aenddoc\formshade\viewkind4\viewscale100\pgbrdrhead\pgbrdrfoot \fet0\sectd \linex0\headery709\footery709\colsx709\endnhere\sectdefaultcl {\*\pnseclvl1\pnucrm\pnstart1\pnindent720\pnhang{\pntxta .}}{\*\pnseclvl2\pnucltr\pnstart1\pnindent720\pnhang{\pntxta .}}{\*\pnseclvl3\pndec\pnstart1\pnindent720\pnhang{\pntxta .}}{\*\pnseclvl4\pnlcltr\pnstart1\pnindent720\pnhang{\pntxta )}}{\*\pnseclvl5\pndec\pnstart1\pnindent720\pnhang{\pntxtb (}{\pntxta )}}{\*\pnseclvl6\pnlcltr\pnstart1\pnindent720\pnhang{\pntxtb (}{\pntxta )}}{\*\pnseclvl7\pnlcrm\pnstart1\pnindent720\pnhang{\pntxtb (}{\pntxta )}}{\*\pnseclvl8\pnlcltr\pnstart1\pnindent720\pnhang{\pntxtb (}{\pntxta )}}{\*\pnseclvl9\pnlcrm\pnstart1\pnindent720\pnhang{\pntxtb (}{\pntxta )}}\pard\plain \s15\nowidctlpar\widctlpar\adjustright \f2\fs20\cgrid {TRANSONIC TURBULENT FLOW CALCULATIONS OVER THE VLS

\par FOREBODY

\par Cesar A. Buonomo & Jo~ao L. F. Azevedo

\par Instituto de Aeron'autica e Espaco, Centro T'ecnico Aeroespacial - CTA/IAE/ASE-N CEP 12228-904 - S~ao Jos'e

\par dos Campos - SP - Brasil - E-mail: azevedo@ase2.iae.cta.br

\par Abstract

\par A detailed study of transonic, turbulent flow conditions over the first Brazilian satellite launcher, the VLS, is

\par described. The emphasis of the work is in developing the capability of accurately simulating realistic rocket

\par flowfields and in understanding the behavior of two turbulence models in such conditions. The flowfields over

\par the VLS at zero angle of attack are modeled by the axisymmetric, thin layer Navier-Stokes equations, and

\par turbulence closure is obtained with the implementation of two different eddy viscosity models. A comparison

\par of the characteristics of these two models is performed. Accurate solutions for the transonic flight regime are

\par obtained. Non-reflective boundary conditions are also implemented, improving the representation of far field

\par boundaries and the code's convergence characteristics in most cases.

\par Keywords

\par Transonic Flow, VLS, Turbulence Modeling, Finite Difference Method, Improved Boundary Conditions.

\par 1. INTRODUCTION

\par A substantial amount of work has been devoted to the development of reliable computational

\par tools for flow simulation in the context of the aerodynamic design of the VLS system, the first

\par Brazilian satellite launcher. The VLS is a four-stage vehicle in which the first stage is composed

\par of four strap-on boosters around a central core. The vehicle has a hammerhead-type payload

\par shroud which is a configuration known to be prone to flow separation during the transonic flight

\par regime. The experience with similar launchers has shown that the payload acoustic excitation

\par loads due to the separated flow condition in the transonic regime can be as large as those

\par experience by the payload at takeoff. Therefore, there is particular importance in having a

\par thorough understanding of the vehicle's forebody flow condition in this regime.

\par Of special interest in the present case, previous work by Azevedo et al. (1995) has performed

\par quite an extensive study of transonic and supersonic flowfields over the VLS at zero angle of

\par attack. In that particular case, the primary concern was the flowfield around the vehicle forebody

\par and the study had been motivated by the need to obtain boundary layer thickness information.

\par The computations performed at that time presented very good agreement with the available

\par experimental data for the majority of the conditions analyzed, except that the transonic case

\par at M1= 0.90 seemed to converge to the wrong shock position as the mesh was refined. The

\par Baldwin and Lomax model (Baldwin & Lomax, 1978) was used for those simulations, and the

\par grid refinement studies were performed by increasing the number of points in the longitudinal

\par direction. Therefore, in this context, the major purpose of the present work is to extend the

\par previous study in order to fully understand the VLS forebody flow characteristics at transonic

\par flight conditions and to further the validation the present flow simulation capability.

\par 1

\par \page

\par }}