COB768	TRANSIENT LAMINAR FORCED CONVECTION HEAT TRANSFER IN CIRCULAR AND PARALLEL-PLATES DUCTS OF HERSCHEL-BULKLEY FLUIDS

E. N.    Macêdo and J. N. N. Quaresma

Chemical Engineering Department – CT/UFPA, Campus Universitário do Guamá, Rua Augusto Corrêa, 01, 66075-900 – Belém-PA-Brasil, E-mail: quaresma@marajo.ufpa.br

The transient analysis of the thermal entry region in laminar forced convection heat transfer of Herschel-Bulkley fluids, within both circular and parallel-plates ducts, is analytically studied through the so-called Generalized Integral Transform Technique (GITT). A prescribed constant uniform wall temperature is considered for the boundary condition. Benchmark results for the local Nusselt numbers in regions very close to the duct inlet are established, for various rheological behavior parameters, such as power-law exponents and yield numbers, which are then confronted with the previous results in the recent literature.

Keywords: Generalized Integral Transform Technique, Transient internal forced convection, Herschel-Bulkley fluids, Parallel-plates channel and circular tube.