COB191	WAVE PROPAGATION IN DAMAGEABLE ELASTO-VISCOPLASTIC BARS

Felipe Bastos de Freitas Rachid & Saulo Guimarães Machado

Departamento de Engenharia Mecânica - Universidade Federal Fluminense - Rua Passo da Pátria, 156 - São Domingos - Niterói / RJ

Cep. 24.210-240

This paper presents a theoretical investigation, based on numerical simulations, of the wave propagation phenomenon in damageable elasto-viscoplastic one-dimensional medium. The degradation of the medium induced by inelastic deformations is described by a local continuum damage model in a context of internal variable theory. To solve the hyperbolic governing equations, the Glimm’s scheme along with a splitting technique were used. Numerical simulations with and without damage are compared so that the influence of the damage on the wave propagation phenomenon is highlighted.

Keywords: Wave Propagation; Continuum Damage Mechanics; Elasto-viscoplasticity; Glimm’s Scheme.