COB1194	FLUXO DE POTÊNCIA ENTRE VIGAS UNIDAS EM L EXCITADAS POR FORÇAS TRANSVERSAIS / POWER FLOW BETWEEN L JOINED BEAMS EXCITED BY TRANSVERSE FORCES

Fábio Fiates1, Arcanjo Lenzi1  e Ernani. Luis. Sztanjbok2

1 - Laboratório de Vibrações e Acústica (LVA) Depto de Engenharia Mecânica, UFSC, cp. 476 - CEP 88040-900 -tel. 048 234 0689

fax 048-234 1519 - Florianópolis - SC, email:   fafiates@mbox1.ufsc.br    	arcanjo@emc.ufsc.br

2 - CENPES - PETROBRÁS - Ilha do Fundão, Rio de Janeiro, RJ, email: ernani@cenpes.petrobras.com.br

Beams joined in L are analyzed by the mobility approach. An initial analysis was carried out to determine what waves and coupling types are responsible for the transfer of most of the input energy to the coupled beam. For out-of-plane point excitations, torsional waves were observed to play na important role in power flow between beams by the flexural-torsional waves coupling. Bending waves modeled by Timoshenko’s theory leads to greater flow of power when compared to Euler’s theory. This is attributed to the larger modal density, and angular and transversal displacements provided by the shear and inertia effects taken into account by Timoshenko’s theory.

Keywords: Fluxo de Potência, Vigas Acopladas, Acoplamento Flexão-Torsão, Método da Mobilidade

Power Flow, Coupled Beams, Flexural-Torsional Coupling, Mobility Approach