COB964	AVALIAÇÃO DE MÉTODOS DE INTEGRAÇÃO NUMÉRICA APLICADA A IDENTIFICAÇÃO DE PERTURBAÇÕES EXTERNAS / NUMERICAL VALUATION INTEGRATION METHODS APPLIED TO A EXTERNAL PERTURBATIONS IDENTIFICATION

Valdemir Mariano

Departamento de Engenharia Mecânica, Centro de Ciências e Tecnologia - UFPB / ETFPE - UNED Pesqueira

Natanael Victor de Oliveira

Departamento de Engenharia Mecânica, Centro de Ciências e Tecnologia - UFPB

CEP 58109-970 - Campina Grande, Brasil - E - mail:jornaljc@cct.ufpb.br

In this work the accuracy of numerical integration methods such as: Central Difference method, Houbolt method, Wilson q and Newmark methods have been investigated. These methods were applied on a process of external perturbations (force) identification in dynamic systems using least square estimation method. The results obtained from the application of each method were compared with the results obtained by using the numerical integration Runge-Kutta method of 4th order. In this study, a MDOF system of two degree-of- freedom, which consists of an axis-plain cylindrical journal bearing system, was applied. From the results it can be conclued that the Newmark method has an accuracy closer to the Runge-Kutta method than the other methods investigated.

Keywords: Avaliação Numérica, Integração, Identificação, Força. - Numerical Valuation, Integration, Identification, Force.