COB877	Simulation of Helicopter Yaw Motion and Blade Rotational Speed Decrease after Tail Rotor Shaft Failure

Olympio Achilles de Faria Mello

Instituto de Aeron‡utica e Espao, Centro TŽcnico Aeroespacial,

12228-904 - S‹o JosŽ dos Campos - SP, Brazil - e-mail: oamello@iconet.com.br

After a helicopter accident with typical tail rotor loss characteristics, an alternate possibility was suggested in that the accident occurred due to pilot's loss of pedal control after he suddenly increased the collective pitch near the ground and delayed applying pedals as needed. In order to investigate this hypothesis, numerical flight dynamics simulations were performed. Simple models of the aircraft yaw dynamics and tail rotor aerodynamics were used. For tail rotor aerodynamics, momentum and blade element theories, supplemented by empirical formulae for vortex ring and turbulent wake states, were used. For tail rotor loss simulations, the blade rotational equation of motion was also established. All equations of motion were time-integrated using a fourth-order Runge-Kutta scheme. Simulation results indicated that pedal control would still be available even with a three second pilot delay in applying the pedals, therefore the hypothesis of loss of pedal control was discarded and the hypothesis of loss of tail rotor was maintained.

Keywords: Helicopter, Simulation, Accident, Tail Rotor.