COB419	DYNAMICALLY  COMPENSATED  CAMS

Luiz Augusto Martin Gonçalves*, Pascal Degezelle & Joris de Schutter

Division of Production Engineering, Machine Design and Automation (PMA), Department of Mechanical Engineering, Katholieke Universiteit Leuven, Celestijnenlaan 300B, B-3001 Leuven, Belgium-E-mail: luiz.goncalves@mech.kuleuven.ac.be

In a cam-follower system the dynamics of the follower-train mechanism cause an actual motion which deviates from the desired one. This effect can be compensated by taking into account the inverse dynamic model of the follower-train in the design of the cam motion law. By considering a constant cam velocity, the follower-train has linear dynamics. However, due to the varying transmission ratio, and due to other nonlinear effects, the whole drive train is a nonlinear system, and nonlinear procedures should be used to fit a motion law. A theoretical analysis with only the linear feedforward compensation, supported by simulation results, has shown the potential of this approach, at least in the case of simple follower-train dynamics: a considerable reduction of the motion error, and a good robustness with respect to errors in the estimated resonance frequency and damping ratio of the follower-train. Experiments with a small driving servomotor, as expected, show different results, due to the non-constant angular velocity. The observed cam angular velocity ripple can be taken into account, in future experiments, to design a complete nonlinear feedforward motion law.

Keywords: Cams, cam vibration, tuned cams, dynamically compensated cams, feedforward compensation.