COB432	AVALIAÇÃO DE DANOS ESTRUTURAIS UTILIZANDO O ALGORITMO GENÉTICO / STRUCTURAL DAMAGE ASSESSMENT USING THE GENETIC ALGORITHM

Mario Antonio Bavaresco, Francisco José da Cunha Pires Soeiro & Cicero Vianna de Abreu

Departamento de Engenharia Mecânica e de Materiais - IME/RJ, CEP 22290-270 Rio de Janeiro, Brasil

e-mail: cvabreu@taurus.ime.eb.br

Departamento de Engenharia Mecânica - UERJ, CEP 20550-013 Rio de Janeiro, Brasil - e-mail: soeiro@uerj.br

The present paper describes an approach for structural damage assessment that has its basis in methods of system identification. Response of a damaged structure differs from that obtained from  the original  and healthy structure. A refined analytical model that predicts correctly the response of the structure is available. The output error approach of system identification is employed to determine changes in the analytical model necessary to minimize differences between the measured and predicted response. Structural damage are represented by  damage variables in which are lumped changes in the stiffness matrices resulting from variations in geometry or material properties of the structure during damage. The design space of the resulting optimization problem is highly nonconvex and has several local optima. In some cases methods of mathematical programming using gradients are not able to detect damage. This work presents a departure from that approach using  one stochastic method of global optimization which is the genetic algorithm. Measurements of static deflections are used in the identification procedure. Principal shortcomings in the proposed approach and methods to circumvent these problems are also discussed.

Keywords: Structural Damage Assessment, Genetic Algorithms, System Identification, Structural Optimization

Avaliação de Danos Estruturais, Algoritmos Genéticos, Identificação de Sistemas, Otimização de Estruturas