COB88	Estabilidade de Arcos segmentados de concreto armado / stability of precast concrete segmental arches

Humberto C. Lima Júnior, Paulo B. Gonçalves & Giuseppe B. Guimarães

Departamento de Engenharia Civil, PUC/Rio

Rua Marquês de São Vicente, 225 - Gávea, CEP: 22435-900 - Rio de Janeiro - RJ

Tel. (021)529-9325-Fax (021)511-1546- E-mail: [humberto, paulo, giuseppe]@civ.puc-rio.br

Segmental arch is a structure made of several straight segments of precast concrete, connected at points belonging to a  curve in the form of an arch which circumscribes the structure. This work presents a study of the effects of height to span ratio, number of segments and initial geometric imperfections on the arch stability. The study was carried out with a computational model based on the finite element method. The model takes into account material and geometric non-linearities, and concrete cracking.

It was found that the optimal height to span ratio value is 0,3, the number of segments affects the behaviour of three-hinged arches and that geometric imperfections can drastically decrease the arch load capacity.

Keywords: segmental arch, reinforced concrete, stability, critical load, limit point and bifurcation point. arco segmentado, concreto armado, estabilidade, carga crítica.