COB831	STRESS STIFFENING EFFECTS ON LAMINATED TIMOSHENKO BEAMS WITH PIEZOELECTRIC ACTUATORS

Donatus C. D. Oguamanan

University of Toronto- Institute for Aerospace Studies, 4925 Dufferin St., Downsview, Ont., Canada M3H 5T6

Sérgio Frascino Müller de Almeida

Instituto Tecnológico de Aeronáutica - Div. Enga Mecânica-Aeronáutica,

12228-900 São José dos Campos - SP - E-mail: frascino@mec.ita.cta.br

Jorn S. Hansen

University of Toronto- Institute for Aerospace Studies, 4925 Dufferin St., Downsview, Ont., Canada M3H 5T6

The effectiveness of using piezoelectric elements to control the mechanical performance of laminated structures by inducing favorable in-plane stresses is investigated. A finite element formulation is presented for the analysis of laminated Timoshenko beams with an arbitrary number of piezoelectric actuators and/or sensors placed along the length of the beam. Von Kàrmàn non-linear strain-displacement relations are used and ideal linear behavior is assumed for the piezoelectric actuation. It is shown that with proper boundary conditions the piezoelectric actuators induce in-plane stresses that may significantly affect the mechanical behavior of the beam. It is shown that the stress stiffening is more pronounced for slender beams. A configuration with a piezoelectric actuator at the top of a slender beam and a sensor at the bottom is analyzed. It is demonstrated that neglecting the stress stiffening effects may lead to large errors in both the static and dynamic analyses of this widely used configuration. Further, the stiffness of the beam can be electrically tuned to a desired value with interesting applications in active structures.

Keywords: Piezoelectric, finite elements, stress stiffening, vibration.