COB 154	ABRUPT-EXPANSION FLOWS OF BINGHAM MATERIALS

Monica F. Naccache & Paulo R. Souza Mendes

Department of Mechanical Engineering

Pontifícia Universidade Catolica, Rio de Janeiro, RJ 22453-900 --- Brazil

Flows of viscoplastic materials through abrupt axisymmetric expansions are important in a number of manufacturing and packaging industrial processes. Depending upon the expansion ratio and Reynolds number, an axisymmetric jet is observed in the flow core, whereas elsewhere the material is almost stagnant. The conservation equations that govern this flow are solved numerically with the aid of a finite volume technique. The material is assumed to behave as a Generalized Newtonian Liquid, with a Bingham-type viscosity function. To handle numerically this viscosity function in a complex flow, the bi-viscosity model has been used. The effect of expansion ratio, Reynolds number and yield stress on the flow pattern and effective cross section area is investigated.

Keywords: Abrupt-expansion flows, Bingham materials.