COB7	EFEITO DO MAGNÉSIO SOBRE A GEOMETRIA DA SOLDA E SOBRE AS CARACTERÍSTICAS ECONÔMICAS DE ELETRODOS AWS E7024/ THE EFFECT OF MAGNESIUM ON THE GEOMETRIC AND ECONOMIC CHARACTERISTICS OF THE AWS E7024 ELECTRODES

Celina Leal Mendes da Silva(1), Vinícius Sales Rocha(2), Estela Surian(3), Jesualdo Pereira Farias(4)

(1)Tv. Apinagés, 973/402 - Bairro Batista Campos - 66045-110 - Belém -Pa., Fones: (091) 224 9883 e (091) 222 0617.

Fax: (091) 241-3415 - e-mail: belucio@supridad.com.br

(2)Campus do PICI - Bloco 714 - DEM - 60455-760- Fortaleza - Ce, Fones: (085) 288 9617

(3)Campus do PICI - Bloco 714 - DEM - 60455-760- Fortaleza - Ce, Fones: (085) 288 9617

(4)Rua Bento Albuquerque, 1550/201, Bairro Papicu - 60190 - 080 - Fortaleza - Ce., Fones: (085) 288 9617 e  (085) 288 9600.

FAX (085) 288 9636 - e-mail: jpf@truenet-ce.com.br

Among the different  kinds of electrodes, the rutilic types are acknowledged as those which allow the easiest operation and handling. Their mechanical properties, however, are inferior to those of basic and cellulosic electrodes. A recent study has shown that adding magnesium to the coating of AWS E7024 rutilic electrodes improves considerably the impact strength  of the welding metal and to reduce the content of diffusible hydrogen. This work attempts to analyse the effect of magnesium on the economic characteristics of rutilic electrodes and also on the geometric characteristics of the experimental welding electrodes. Derived from the  AWS E7024 class were developed, with 3.25 mm diam., and exhibiting five different amounts of metallic magnesium ( from 0 to 4% ). Simple deposition automatic weldings in plane position with current DC+, DC-  and AC were performed in 170x50x6 mm ABNT 1020 all-weld metal, using a computerised device. The results obtained were compared using the ANOVA method, and showed that magnesium content does not  effect welding geometry neither economic characteristics of consumables.

Keywords: electrodes,magnesium,welding,rutilic/eletrodos, magnésio, solda, rutílico