COB13	APLICAÇÃO DE VISÃO POR COMPUTADOR PARA GUIAGEM DA TOCHA EM UMA CÉLULA DE SOLDAGEM / COMPUTER VISION APPLICATION ON TORCH GUIDANCE IN A WELDING CELL

Lélio Ribeiro Soares Jr, Víctor Hugo Casanova Alcalde & Sadek Absi Alfaro

Grupo de Automação e Controle, Faculdade de Tecnologia, Universidade de Brasília - UnB - CEP 70.910-900 - Brasília, DF;

E-mail: casanova@guarany.cpd.unb.br

A computer vision system is proposed for guiding the welding torch in a robotic welding station. The vision sensing system consists of a laser-emitting diode, a CCD video camera, a frame grabber card and a PC computer. The camera and the laser device are fixed to the welding torch. The laser beam produces a luminous stripe across the welding path and in advance of the torch. By capturing images from the workpieces reflecting the laser stripe an image processing algorithm establishes a welding reference path and compares it with the actual path. A trajectory deviation error is generated and transmitted to the robot control unit for the necessary action. Provisions for the implementation of a complete visual servo control for the robotic welding station are also discussed.

Keywords: Computer Vision Guidance; Welding Seam Tracking; Robotic Welding Station; / Guiagem por Visão Computacional; Rastreamento de Trajetória de Soldagem; Estação Robótica para Soldagem.