COB161	ANÁLISE COMPARATIVA DE ESTIRAMENTO IN-PLANE VERSUS OUT-OF-PLANE/COMPARATIVE ANALYSIS OF IN-PLANE VERSUS OUT-OF-PLANE STRETCHING

Fernando Ferreira Fernandez & Hazim Ali Al-Qureshi

Departamento de Tecnologia, Instituto Tecnológico de Aeronáutica- ITA-CTA

CEP 12228-900 São José dos Campos, Brasil- E-mail: hazim@mec.ita.cta.br

The limit strains, or the maximum plastic strains that a sheet can support up to the necking point, admitting free deformation on the sheet plane (In plane strains-IP) are quite different of the biaxially stretching operations with spherical punch or pressure (Out of plane strains-OP). The last one, which represents the real industrial  operations, are greater than the IP strains, for the same degree of biaxial tension. The main scope of this work is to perform stretch operations IP, using a specially designed device able to input IP strains, and verify if differences between theoretical models and experimental data are based on the fact the stretching operation be IP or OP.

Keywords: Chapas metálicas, Estiramento Biaxial, Deformações limites. Sheet metals, Biaxially stretching, Limit strains