# COB1084	ASPECTOS TECNOLÓGICOS DO DESENVOLVIMENTO DA FURAÇÃO POR ESCOAMENTO / TECHNOLOGICAL ASPECTS ON THE DEVELOPMENT OF THE FLOWDRILLING

Walter Lindolfo Weingaertner, João Carlos Oliveira Lopes, Daniel Carlos da Silva, Otacílio Cardoso Duarte Filho, Cristiano Rafael Schramm, Gerson Ismael de Souza, Manfred Molz

Laboratório de Mecânica de Precisão - EMC – UFSC, Campus Universitário Trindade - CP 476 EMC, Florianópolis - SC , E-mail :

# Franz Tikal, Roland Heiler

Instituto für Produktionstechnik und Logistik - IPL – GhK, Kurt-Wolters-Str. 3 - 34 125, Kassel – Deutschland,

E-mail : heiler@hrz.uni-kassel.de

Flowdrilling is a relative new manufacturing process for bush forming. A cemented carbide tool with special geometry is used on standard drilling machines to produce bushes in thin metal sheets, without chip removal. After the drilling process a cold forming tap is used to generate the thread in these bushes by plastic deformation. A threaded bush is then used like a sheet metal joint in many applications. This work is about new research topics which have been developed in a co-operation between the Laboratório de Mecânica de Precisão/UFSC, from Florianópolis - Brazil, and the Institut für Produktionstechnik und Logistik/GhK from Kassel – Germany. These topics are concerned about the deformation process of the sheet material, development of new materials for the tools, microstructure analysis of the sheet material and development of some applications of this technology.

Keywords: Furação por escoamento, rosqueamento por conformação, deformação, materiais, chapas metálicas.

Flowdrilling, flowtapping, deformation, materials, sheet metal.