COB585	APLICAÇÃO DE BROCAS HELICOIDAIS COM AFIAÇÕES ESPECIAIS NA FURAÇÃO DE LIGA DE TITÂNIO / APPLICATION OF TWIST DRILLS WITH SPECIAL POINTS TO DRILLING TITANIUM ALLOY.

# u Olívio Novaski & ¨ Carlos Cesar de Castro Deonisio

u Departamento de Engenharia de Fabricação, Faculdade de Engenharia Mecânica – UNICAMP - CP 6122 – CEP 13083-790 – Campinas, SP, Brasil

¨ Centro Técnico Aeroespacial – CTA / IAE/ASA – P, Praça Mal Eduardo Gomes, 50, Vila das Acácias, CEP 12228 – 904, São José dos Campos – SP, Brasil.

With the development and application of new materials, the manufacturing processes needs to be optimized through the improvement of the geometry and material of the tools and the use of the adequate manufacturing parameters. The objective of this work is to study the drilling process of the Ti-6Al-4V alloy, which represents 45% of the total production of titanium and its alloys, and is heavily used in the aerospace, chemical, maritime, electronic and biomedical industries. For this purpose, the objective of this work concentrate on the selection of the adequate tool point geometry for the manufacturing of Ti-6Al-4V alloy with high speed steel twist drills. Tests with various tool point geometries were done and the behaviour of the tools was observed regarding cutting forces, tool wear, form of the chips and dimensional deviation of the hole. The results show that the “Bickford”, NAS-P3 e “Helical” are the best choice for machining Ti-6Al-4V alloy.

Keywords: Drilling/Furação; Titanium/Titânio; Twist drill/Broca helicoidal; High speed steel/Aço rápido; Machining/usinagem.