COB1088	DESENVOLVIMENTO DE UMA RETIFICADORA PARA A USINAGEM DE LIMAS ENDODÔNTICAS

Walter L. Weingaertner, Durval João De Barba Jr.

Laboratório de Mecânica de precisão, Departamento de Engenharia Mecânica - UFSC, CEP 88010 - 970 Campus Universitário, Trindade, Florianópolis - SC Caixa Postal 476, E-mail: lmp@labmat.ufsc.br

This paper shows the conceptual project and the functional model conception of a machine tool for the grinding of root canal files type hedströem. Initially there is a brief look on the reamers characteristics for the determination of the grinding process kinematics, so to determine a pre-conception. After that, through QFD, a classification of the project specifications for the machine is obtained, which combined with its functions structure, determines the grinder conception. To prove that the grinding process for obtaining reamers is proper for the small dimensions of the work piece (less than 1 mm in diameter), a functional model is set.

Keywords: Retificação de pequenos diâmetros, projeto conceitual, estrutura de funções, modelo funcional.