COB608	PROCESSO DE OBTENÇÃO DE COMPOSTOS INTERMETÁLICOS DE Ti E Al UTILIZANDO ONDAS DE CHOQUE / PROCESS OF OBTANING INTERMETALLIC COMPOUNDS OF Ti AND Al BY USING SHOCK WAVES

Arnaldo Ferreira e João Edison Minnicelli

Departamento de Engenharia Mecânica e de Materiais, Instituto Militar de Engenharia - IME.

Praça General Tibúrcio 80, 22290-270, Praia Vermelha, Rio de Janeiro, RJ - Brasil

Elemental powders of Ti and Al were mixtured and, by employing the flyer plate technique, experiments were performed in order to determine the threshold chemical reaction between the powders, and to obtain compacts that allow to characterize the reagents compacted by shock wave.    Different proportional mixtures between Ti and Al were used, as an attempt of obtaining improved reactions and better compaction of the materials. The experimental parameter that was varied was explosive quantity, increasing the amount of energy imparted to the powder material. The intermetallic compound TiAl was also mecanically mixtured with Ti and Al powders, in order to compare the compacted material. Analysis by X-Ray diffraction and Scanning Electron Microscopy were performed to determine the chemical changes during the compression by shock waves. Successful consolidation was obtained and was comproved by strong bonding between individual particles. The    main parameter that is significant during the process of chemical reaction and the experimental results are presented and discussed.

Keywords: Dynamic Consolidation; Shock Waves; Shock-Induced Chemical Reactions; Ondas de Choque; Metalurgia do Pó.