COB121	FABRICAÇÃO E CARACTERIZAÇÃO DE FERRAMENTAS DE CORTE DE Si3N4 / MANUFACTURING AND CHARACTERIZATION OF TOOLS OF THE Si3N4

Olivério Moreira de Macedo Silva, Cosme Roberto Moreira da Silva & Sandro Aparecido Baldacim

Divisão de Materiais-Instituto de Aeronáutica e Espaço-Centro Técnico Aeroespacial-Amr/Iae/Cta

Cep 12228/904 - São José dos Campos - SP - Brasil

Silicon nitride based ceramic cutting tools were developed in Brazil. Ceria and aluminium nitride were used as sintering aids. The samples evaluations were performed using especific mass determination, X ray diffraction, microhardness testing, fracture thoughness by indentation method and microstructural examination via scanning electron microscopy. After samples evaluation, some pressureless sintering silicon nitride based ceramic cutting tools were made. A piece of cast iron was machined using these cutting tools. For the purpose of comparison, a commercial hard metal cutting tools was also  used to perform the same machining.

Keywords: Sintering,  tools, silicon nitride, ceria, aluminun nitride / Sinterização, ferramentas, nitreto de silício, céria, nitreto de alumínio