COB033	INFLUENCE OF THE NUMBER OF SPECIMENS IN THE REFERENCE TEMPERATURE DETERMINATION - INITIAL APPROACH / INFLUÊNCIA DO NÚMERO DE CORPOS DE PROVA NA DETERMINAÇÃO DA TEMPERATURA DE REFERÊNCIA - ABORDAGEM INICIAL

Carlos Alexandre de. J. Miranda1  & John D. Landes2

1IPEN-CNEN/SP, SP, Brazil, e-mail: cmiranda@ipen.net.br;

2University of Tennessee at Knoxville, USA; e-mail: John-Landes@utk.edu

The ASTM E08.08 subcommittee is preparing a new standard, currently in the draft version #13, to determine the reference temperature To for ferritic steels in the transition region. The minimun number of valid experimental results in the data set, to be analyzed by this proposed standard, for a given temperature T, is six. In this work this assumption (six results) is verified by comparison among the results obtained, in terms of T-To, with different number of toughness values (subsets) taken from a given large set that fits perfectly the Weibull three-parameter probability distribution. The conclusion is an indication of the way to be followed to determine the minimum number of experiments/results necessary to obtain at least 90%, 95% or 98% of the possible T-To values near the target one (that of the whole set from which the subsets are taken)

Keywords: Fracture Mechanics, Transition, Reference Temperature, Reliability

Mecânica da Fratura, Transição, Temperatura de Referência, Confiabilidade