COB390	UMA ANÁLISE DO COMPORTAMENTO À FRATURA DE AÇOS ESTRUTURAIS MICROLIGADOS UTILIZADOS EM APLICAÇÕES ÁRTICAS

Celso P. M. Pereira*, C. Moura Branco** &  A. Sousa e  Brito**

* Faculdade de Engenharia de Guaratinguetá - FEG/UNESP, CEP 12500-000  Guaratinguetá-SP, Brasil - Fax 012 525 24 66

** Departamento de Engenharia Mecânica, Instituto Superior Técnico - ICEMS/IST

Av. Rovisco Pais, 1  -  1096  Lisboa Codex , Portugal  -  Fax 01 847 40 45

The fracture toughness behaviour of four microalloyed structural steel, with different composition and process routes was evaluated  for artic applications. CTOD three point bend tests were carried out at a range of temperatures between +10ºC and  -110ºC in HAZ of submerged arc butt welded specimens.  Microstructural analysis and hardness (HV) profiles mensurement were also carried out to complement  the  mechanical tests. The results show that all the four steels can work at -50ºC and withstand a CTOD  value in excess of  0.2 mm .

Keywords: Fracture Toughness,  Microalloyed Steels,  Weld  Microstructures, Welding and  HAZ  Properties.