COB221	MOTION CONTROL OF A BIAXIAL MACHINE TOOL USING A VERSATILE CROSS-COUPLING CONTROLLER / CONTROLE DE MOVIMENTOS DE UMA  MÁQUINA-FERRAMENTA BIAXIAL USANDO UM ALGORITMO COM EIXOS ACOPLADOS

Helder Barbieri Lacerda(*) & Eduardo Morgado Belo(* *)

(*) Fabrication and Machine Design Department, Federal University of Santa Maria

CEP 97.119-900 - Santa Maria, RS, Brazil - E-mail: helderbl@sc.usp.br

(* *) Mechanical Engineering Department, EESC, University of São Paulo

CEP 13.560-250 - São Carlos, SP, Brazil - E-mail: belo@sc.usp.br

This work deals with a type of cross-coupling controller applied to control the machine tool biaxial table motions. A special algorithm was created to perform well any kind of trajectory, linear and non-linear with constant or varying  curvature radius and corners. There is no need to make any previous adjustments. It utilizes information from the interpolator and axial positioning errors to calculate the instantaneous radius and curvature centre of the trajectory and also, the contour error. Moreover, it is able to change the proportional, integral and derivative gains of the control law, according to the kind of path. It can be put to work together with the machine axial controllers, helping them. The system is simulated using a non-linear machine model which includes friction, backlash, cutting forces and the ballscrew elasticity. The response results are compared with those of common PID's in order to analyse the effectiveness of this controller over the system.

Keywords: machine tools, cross-coupling motion controllers / máquinas-ferramenta, controladores de movimento com eixos acoplados