COB679	MODELING, EXPERIMENTAL IDENTIFICATION AND LQG/LTR CONTROL OF A MULTIBODY SYSTEM WITH FLEXIBLE APPENDAGES

Álvaro Manoel de Souza Soares, Luiz Carlos Sandoval Góes & Luiz Carlos Gadelha de Souza

Depto de Informática - Universidade de Taubaté - UNITAU

Av. Marechal Deodoro da Fonseca, 605 - 12100 - 000 - Taubaté - SP - E-mail : alvaroms@embratel.net.br

Divisão de Engenharia Mecânica-Aeronáutica - Instituto Tecnológico de Aeronáutica - ITA-CTA - IEMP

12228-900 - São José dos Campos,  SP - E-mail : goes@mec.ita.cta.br

INPE - Instituto Nacional de Pesquisas Espaciais

Av. dos Astronautas,1758 - C.P. 515 - 12201-970 - S.J.Campos, SP, Brasil - E-mail : gadelha@dem.inpe.br

This work shows simulational results of control using the LQG/LTR technique. The system used was a multibody structure composed by a hub and two flexible appendages made of aluminum which are linked to a DC motor and represents a satellite like structure. The model used was obtained trough the Lagrangian approach and the model discretization was done by the assumed modes method. An experiment assembled in the dynamical laboratories of ITA (Instituto Tecnológico de Aeronáutica) was used to validate the analytical model. The experimental set up has four kinds of sensors available. One accelerometer in the tip of one of the aluminum beams, two strain-gages located in known points of the beams, one tachometer signal which is provided for the DC motor and a potentiometer which measures the hub angular position. A modal analysis of the system is shown and the experimental results are used to validate the analytical model. To control this system a LQG/LTR technique was employed. This kind of control technique is suitable for systems like this one that have some uncertainties due, for example, model discretization. The aim of the control is to drive the hub to an desired angular position. The results of behavior of the closed loop system with a reference trajectory are also shown.

Keywords: Flexible structures, Multibody system, Modal analysis, Modeling, Position Control