COB 1332	INTERAÇÃO DO SISTEMA DE CONTROLE E A ESTRUTURA FLEXÍVEL DE UM SATÉLITE DURANTE TRANSFERÊNCIA ORBITAL / INTERACTION BETWEEN CONTROL SYSTEM AND FLEXIBLE STRUCTURE OF AN SATELLITE DURING ORBIT TRANSFER.

Adenilson Roberto da Silva, Luiz Carlos Gadelha de Souza

### Departamento de Mecânica Orbital e Controle, Instituto Nacional de Pesquisas Espaciais – INPE

### São José dos Campos – CEP 12227 – 010 São José dos Campo, Brasil - adenilson@dem.inpe.br - gadelha@dem.inpe.br

In this paper, the interaction between the attitude control system and the flexible structure of an artificial satellite during orbit transfer maneuvering has been investigated. The satellite was modeled by a rigid central body with one or more flexible appendages. The dynamics equations were obtained by a Lagrangean approach. The flexible appendages were treated as a clamped-free beam and its displacement was discretized by assumed-mode method. In the satellite transfer maneuver a typical Hohmann procedure and a burn-coast-burn strategy were used, the attitude was controlled by on-off controller. As a result, it was verified that if any jet fire cycling near the fundamental frequency of flexible appendages, a possibility of the interactions between control system and flexible structure can  exist which could damage the performance of control system. Therefore, as security, in the control system design a bandwidth with one decade below of first natural frequency of the flexible structure should be used.

Keywords: Estruturas Flexíveis, Sistemas de Controle, Transferência de Órbita / Flexible structures; Control Systems; Orbit Transfer