COB422	AUTOMAÇÃO DE UM SISTEMA DE RAMPA para EMBARQUE E DESEMBARQUE DE VEÍCULOS EM NAVIOS/ Automation of a ramp system for ship loading and unloading of vehiches

Diolino J. Santos Filho & Paulo Eigi Miyagi
Departamento de Engenharia Mecânica/Mecatrônica, Escola Politécnica da USP - CEP:05508-900, São Paulo, SP, Brasil

E-mail: diolinos@usp.br    pemiyagi@usp.br

When do not exist a special port infrastructure, the most common way to realize the access of vehicles in ships is trough a ramp system. One alternative to improve the efficiency of  this system is trough techniques of task automation. This paper approach this problem through the application of a methodology for Event Driven Systems design based on Petri nets. At first, a conceptual model of the system is generated by using PFS technique (Production Flow Schema). Then, the dynamic behavior of the system is described by a flow of tokens in oriented graphs called MFG (Mark Flow Graph). Through a gradual refinement and interpretation procedure a detailed and consistent representation of the control strategies is obtained. Validation of the control strategies in the present case were carried out through simulation analysis. Finally, the resulting control strategy model is translated to an adequate language of programmable controllers. The overall tests were carried out in a prototype model of the ramp system.

Keywords: Discrete Event Dynamic Systems, Task automation, Petri nets, Production Flow Schema, Mark Flow Graph