COB649	SOME COMMENTS ON DISPLACEMENT MEASUREMENTS USING ACCELEROMETERS

Ribeiro, J.G.T.

Materials and Mechanical Engineering Department , IME, CEP 22.290-270  Praça General Tibúrcio 80, Rio de Janeiro, R.J., Brazil

E-.mail: d4jose@epq.ime.eb.br

Castro, J.T.P. & Freire, J.L.F.

Mechanical Engineering Department, PUC-Rio, CEP 22.453-900  R. Marquês de São Vicente 225, Rio de Janeiro, Brazil

E-mail: jtcastro@mec.puc-rio.br    &   jlfreire@mec.puc-rio.br

Some  problems in measuring displacements using analog double integration of acceleration data are discussed in this paper. The main problem in using analog double integration is due to the nonlinear phase response of the double integrator and its transient response that can induce great errors. These errors are unacceptable when the purpose of the measurement is to verify structural integrity, since damage is dependent on the peak and valleys of the waveform. In order to quantify some of  these errors, several displacement measurements were made using simultaneously double integration of acceleration data and a LVDT. These results indicate that accelerometers should not be used to measure displacements unless the double integration errors are considered and appropriately corrected.

Keywords: Measurements, accelerometers, displacements, integration/ Medições, acelerômetros, deslocamentos, integração