COB1278	RESPOSTA DINÂMICA DE SENSORES DE TEMPERATURA ATRAVÉS DO DESMASCARAMENTO EM MEIO LÍQUIDO

João Nildo de S. Vianna, Alessandro Borges de S. Oliveira & Arthur Carneiro Neto

Departamento de Engenharia Mecânica, Universidade de Brasília - ENM-UnB

CEP 70910-900 Brasília - Brasil - E-mail: vianna@enm.unb.br

This work presents a comparative study between two methods for the generation of a step function temperature signal to be used in the dynamic calibration of temperature sensors in liquid. The classic method, in which the sensor is rapidly immersed in a fluid at different temperature, is compared with the unmasking method. In this method the sensor is placed in the fluid, but is isolated from it by a mask pressurized internally. An especially designed rotating tank was used in these tests. The results obtained have shown that, at least for a given type of sensor, the unmasking method presents better results than the immersion method.

Keywords: Calibração Dinâmica, Tempo de Resposta,  Sensores de Temperatura, Medição Transiente de Temperatura.