COB453	CARREGAMENTO DINÂMICO DE MANCAIS RADIAIS COM CAVITAÇÃO DO FILME DE ÓLEO / DYNAMIC LOADING OF JOURNAL BEARINGS WITH OIL FILM CAVITATION

Evandro de Souza Santos

Schulz S. A. - Pesquisa e Desenvolvimento, Rua Dona Francisca 6901 - Distrito Industrial

89219-000 Joinville, SC - E-mail: schulz@netville.com.br

Alvaro Toubes Prata

Departamento de Engenharia Mecânica, Universidade Federal de Santa Catarina

88040-900 Florianópolis, SC - E-mail: prata@nrva.ufsc.br

Hydrodynamic lubrication of journal bearing is governed by the Reynolds equation. The main difficulty in using Reynolds equation resides in the precise determination of the edge or the angular position after which the oil cavitates and starts to flow into a series of streamers. One of the objectives of the present work is to revise and compare some of the numerical algorithms encountered in the open literature to deal with the cavitation phenomena. Emphasis is also placed on selection of the coordinates system used to analyze the problem. Usually the hydrodynamic lubrication problems have been studied based on Reynolds equation referred to a rotating coordinates system where one of the coordinates is coincident with the centerline of the journal bearing. Here, the integration of Reynolds equation is performed with respect to an inertial coordinates system, and this is a novelty. The numerical results were obtained from both static and dynamic loading under conservative and non conservative boundary condition applied to the cavitation front. The Reynolds equation in fixed coordinates system performed well, and the Elrod Algorithm proved to be the most effective method for dynamic loading calculations.

Keywords: Cavitation in bearings, lubrication, dynamic loading, Reynolds equation in fixed coordinates system.

Cavitação, lubrificação, carregamento dinâmico, equação de Reynolds em sitema de coordenadas fixo.