COB267	Sealing Zone Conditions of Radial Lip Seals Considering the Viscoelastic Properties of Lubricating Fluids

Kay Wiehler, Sylvio Jose Ribeiro de Oliveira & Manfred Vötter

Technische Universität Hamburg-Harburg, Konstruktionstechnik II, 21071 Hamburg, Germany, E-mail: wiehler@tu-harburg.de

The proceedings in the sealing zone between a lip seal and shaft surface are still not thoroughly understood. Due to the small fluid film thickness (micrometer range), the high temperatures (up to 200°C), the shear stress (MPa range) and the shear rate (> 106 s-1) it is difficult to achieve accurate measurements. A recently developed model takes the non-newtonian fluid behavior into account in order to describe the existence of a fluid film in the sealing zone. The radial pressure of the seal is compensated by the viscoelastic properties of the fluid and no deviation from parallel of the surfaces in micro and macro range is mandatory for this model. The measurement of rheological data of lubricating fluids at high shear rates is difficult and reliable data is hardly accessible. Therefore the data of higher viscous polymer melt and solution experiments is taken to model the viscoelastic behavior of lower viscous lubricating fluids. This simulation shows for various working conditions the influence of the fluid properties upon fluid film thickness, torque and pumping action of radial lip seals.

Keywords: Radial Lip Seal, Non-Newtonian Fluid, Pumping Mechanism, Fluid Film Thickness