COB902	OTIMIZAÇÃO APLICADA À CONFIGURAÇÃO E OPERAÇÃO DE SISTEMAS DE COGERAÇÃO / SYNTHESIS AND OPERATION OPTIMISATION OF COGENERATION SYSTEMS

Flávio Neves Teixeira, André Ramon Silva Martins & Luiz Augusto Horta Nogueira

Instituto de Engenharia Mecânica, Escola Federal de Engenharia de Itajubá - EFEI

CEP 37.500-000, Itajubá, Minas Gerais, Brasil - E-mail: horta@iem.efei.rmg.br

A methodology to specify and operate optimally cogeneration plants is presented, aiming to lower the cost for  energy production, attending electric power, heating and cooling demands. The general configuration adopted for the cogeneration system includes prime mover, heat recovery steam generator, process boiler, conventional and absorption cooling systems and electricity supply from utility. The equipment capacities and operational policy for the cogeneration plant are determined simultaneously in order to minimise the pay back time. It is presented the modelling of thermal (low and high temperature) and electric loads, taking into account constant demand in 8 periods of time over the year and according the periods used by utilities to price energy. It is developed the modelling of prime movers, using lineal approach to simulate the dependence performance and useful heat availability on the load factor, allowing to consider real commercial equipment. A case study is solved for a shopping center, in order to demonstrate the validity of the proposed method.

Keywords: Cogeneration, Optimisation of Thermal Systems, Gas Turbines, Alternatives Engines

Cogeração, Otimização de Sistemas Térmicos, Turbinas a Gás, Motores Alternativos