COB489	Otimização do Posicionamento de Atuadores Piezocerâmicos em Estruturas Flexíveis Usando um Algoritmo Genético

Virgílio M. da Costa e Silva (*)      &      José Roberto de França Arruda (**)

(*) Departamento de Tecnologia Mecânica, CT/UFPB, CEP: 58059-900, João Pessoa, PB, Brasil - (**) Departamento de Mecânica Computacional, Faculdade de Engenharia Mecânica, UNICAMP, CEP: 13081-970, Campinas, SP, Brasil,    E-mail: arruda@fem.unicamp.br

In this work, a method to determine the optimal location of active elements in structures using a genetic algorithm technique involving discrete variables is presented.    A finite element formulation is used for modeling the dynamics of the structure for both passive and active elements, and an optimization algorithm minimizes the dissipation energy with an optimal control designed using the LQR theory. The optimal location of active members is treated in terms of (0,1) variables.    Numerical results obtained    for a simply supported beam with two piezoceramic actuators are presented to illustrate the actuator placement optimization method. The obtained results show that the genetic algotithm    can be sucessfully applied to combinatorial optimization problems such as the actuator placement problem in scope.

Keywords: Posicionamento de Atuadores, Atuadores Piezocerâmicos, Algoritmos Genéticos.